SECTION I - PRODUCT IDENTIFICATION

TRADE NAME:	Hartco Green Cleaner                    	 PRODUCT CODE: #1162

CHEMICAL NAME: Alkaline cleaner                        Date: December 7, 2006

CAS #: Mixture, no single number applies                  DOT HAZARD CLASS: N/A

DOT SHIPPING NAME: Cleaning compound liquid, Non Hazardous

SECTION II - HAZARDOUS INGREDIENTS

HAZARDOUS INGREDIENTS        %	CAS #                          TLV (UNITS)                         PEL

Alkyloxypolyethyleneoxyethanol    <1 	9016-45-9	        ND	                                          ND

Sodium Tripoly Phosphate              <1    	7758-29-4	       15 mg/m3		           15 mg/m3

Sodium Metasilicate	     	<1          6834-92-0	        15 mg/m3                            15 mg/m3

SECTION III - PHYSICAL DATA

BOILING POINT:  (F) 230             SOLUBILITY:  Complete               pH: 10.25

PERCENT VOLATILE: N/A           VAPOR DENSITY: N/A                VAPOR PRESSURE: N/A

EVAPORATION RATE (BUTYL ACETATE = 1): 1                            SPECIFIC GRAVITY: 1.06

APPEARANCE & ODOR: Green liquid, lemon fragrance

SECTION IV - FIRE AND EXPLOSION HAZARD INFORMATION

FLASH POINT: Non flammable

FLAMMABLE LIMITS: N/A           LOWER: N/A                                  UPPER: N/A

FIRE/EXPLOSION HAZARDS: None

EXTINGUISHING MEDIA: Foam, CO2, dry chemical

FIRE FIGHTING PROCEDURES: Fire fighters should wear self-contained breathing apparatus and full protective clothing.  Use water spray to cool nearby containers and structures exposed to fire.

SECTION V - HEALTH HAZARD INFORMATION

PERMISSIBLE EXPOSURE LEVEL: ND

EYES: Will cause irritation and redness.

SKIN:  Prolonged contact will cause irritation and redness.

INHALATION: Prolonged inhalation of mist will cause nasal and respiratory irritation.

INGESTION: May cause gastrointestinal irritation.

THIS MATERIAL IS NOT LISTED AS A CARCINOGEN BY IARC, NTP OR OSHA

HMIS HAZARD CLASS: Health = 1       Flammability = 0      Reactivity =0        Other =NA

Ranking: 0 = Least 1 = Slight 2 = Moderate 3 = High   4 = Extreme

FIRST AID

EYE CONTACT: Flush with water for 20 minutes lifting eyelids.  Get medical attention immediately

SKIN CONTACT: Wash thoroughly with water and treat with any moisturizing cream.

INHALATION: Remove victim to fresh air.  Administer artificial respiration if breathing ceases.  Get medical attention immediately.

INGESTION: Do not induce vomiting.  Dilute by giving lots of water.  Give milk of magnesia.  Call physician immediately.

SECTION VI - REACTIVITY INFORMATION

STABILITY: Stable

REACTIVITY: None will occur

HAZARDOUS DECOMPOSITION INGREDIENTS: None

HAZARDOUS POLYMERIZATION: Will not occur

SECTION VII - SPILL, LEAK AND DISPOSAL PROCEDURES

SMALL SPILLS: Mop, sweep, or absorb, and dispose of in accordance with federal, state, and local regulations.

LARGE SPILLS: Mop, sweep, or absorb and dispose of in accordance with federal, state, and local regulations.

DISPOSAL: Reduce PH with vinegar or other weak acid and discharge to sewer.

SECTION VIII - SPECIAL PROTECTIVE EQUIPMENT

EYE AND FACIAL: Wear chemical splash goggles.

SKIN: Vinyl, rubber or nitrile gloves should be used.

RESPIRATORY: Wear OSHA approved mists respirator.

VENTILATION: Mechanical ventilation should be provided.

OTHER:  Wear protective apron and boots.

SECTION IX - SPECIAL PRECAUTIONS

STORAGE: Avoid freezing. Avoid strong mineral acids.

HANDLING: Store in cool dry well ventilated place

LEGEND: ND, Not Determined; N/A Not Applicable; NA, Not Available.

NOTICE

All information appearing herein is based upon data obtained from chemical manufacturers and or recognized technical sources.  While the information is believed to be accurate, Snyder Manufacturing Corporation makes no representations as to its accuracy or sufficiency.  Conditions of use are beyond Snyder Manufacturing Corporation’s control and therefore users are responsible for and assume all risks of their use, handling, and disposal of the product, or from the publication or use of, or reliance upon, information contained herein.  This information relates only to the product designated herein, and does not relate to its use in combination with any other material or in any other process